- -Prepare the following reagents before beginning any reactions

  - Reaction buffer (amounts given for a 250 mL stock):

    - 50 mM MOPS (pH to 7.4 with NaOH) (2.616 g)

    - 1 mM DTT (250 µL of 1 M stock)

    - 100 mM NaCl (1.461 g)

    - 20 mM NaF (0.21 g)

    - 1 µM CaCL2 (36.7 µg, or 249.8 µL of a 1 mM stock)

  - Reaction initiator (amounts given for a 1 mL stock)

    - 50 mM MgCl2 (.01017 g of hexahydrate)

    - 1 mM ATP (100 µL of a 10 mM solution, provided)

    - Prepared in Reaction buffer

  - Micelle mix (make preparation fresh for each plate if possible)

    - 10 mM 18:1 DG (dried under N2 if in CHCl3)

    - 8 mM DOPS (dried under N2 if in CHCl3)

    - .3 % Triton X-100

    - Sonicate twice for .5 seconds each. Solution should turn slightly opaque.

    - Add reaction buffer to get the requisite concentrations (3x of the triton buffer added)

  - Lysis buffer (amounts given for 250 mL stock)

    - 50 mM HEPES (pH 7.2 with NaOH) (2.979 g)

    - 150 mM NaCl (2.192 g)

    - 5 mM MgCl2 (2.541 g of hexahydrate)

    - 1 mM DTT (250 µL of 1 M stock)

    - 1 mM Phenylmethylsulfonyl Fluoride (.044 g)

  - Complete lysis buffer (prepare as needed)

    - 1 Pierce Protease inhibitor tablet, EDTA free (Not Protease/phosphatase tab!)

    - 10 µL Phosphatase Inhibitor Cocktail II

    - Lysis Buffer to 10 mL

    - Vortex and sonicate to mix. You can also pre-crush the tablet before adding

Preparation of Lysates

- Add 500 µL of Complete lysis buffer to the pellet from a 10 cm dish. Dissolve pellet gently, and then sonicate three times for ~1 second each time.

- Spin down pellet at 400xg for 5 minutes and recover supernatant. Supernatant will likely still have noticeable debris, this is fine.

- Check concentration with DC assay and dilute lysate to 1 mg/mL.

Assay

*when measuring inhibitor dry stocks for DMSO solutions, do not try to measure less than 2 mg of compound. The error from measuring small amounts of compound can drastically affect the final concentrations tested in your assay.

- When all of the above solutions are prepared, begin the plate prep. Turn on the incubator heat to 30 degrees.

- Use a half-area 96 well black plate for the assay, and clear plates for reagent prep. Add 1 µL of either DMSO or inhibitor in DMSO to each of the wells in the black plate as needed. Be sure to keep track of which wells you are using.

- Add Micelle mix to the lysate in a 20:1 ratio (i.e. 95 µL mix to 5 µL lysate) or 10:1 (i.e. 90 µL mix to 10 µL lysate) depending on desired signal. You should prepare at least 30 µL of this mix for each reaction you are going to perform.

- Take a new clear 96 well plate and pipet ~24 µL of the lysate mix into each of the appropriate wells. You can also mix the lysate and micelle mix directly in each well, though this is only recommended for when there are few repeats of a single lysate, such as for mock or denatured wells.

- Pipet 19 µL of the lysate mix into each of the inhibitor wells in the black plate with a multichannel pipet.

- Cover plate with saran wrap and place in incubator for 30 minutes.

- When it is close to 30 minutes, pipet N*5+10 µL of the reaction initiator into each of the appropriate wells of a clear plate, where N is the number of wells in that column of the plate (for instance, for a simple run with 2 wells in a column, you’d pipet 20 uL. For four wells in a column, 30 uL, etc).

- Remove the ADP-glo reagent from the freezer to thaw.

- At 30 minutes, get the plate from the incubator and pipet 5 µL of the reaction initiator into each reaction well with the multichannel pipet. Mix thoroughly, then pipet 5 uL of this mix into new wells. These new wells are the final reactions. Re-cover the plate and place back in the incubator for 30 minutes.

- By close to 30 minutes, the ADP-glo reagent should be unfrozen. If not, hand warm it. Pipet N*5+10 µL into each of the appropriate wells of the clear plate. When 30 minutes have elapsed, remove the plate from the incubator and pipet 5 µL of reagent into each reaction well (the wells with 5 uL of the reaction) with the multichannel pipet. Re-cover the plate and leave at room temperature for 40 minutes. Remove the Kinase Detection Reagent from the freezer and store at RT but out of the light.

- By the end of 40 minutes, the Kinase Detection Reagent should be thawed. Pipet N*10+10 µL into each of the appropriate wells of a clear plate. At 40 minutes, add 10 µL to each reaction with the multichannel pipet. Re-cover the plate, and store out of the light at RT for 40 minutes.

- At the end of 40 minutes, remove saran wrap and place in the Clariostar. Read on luminescence (no filter) with a 1 sec integration time.