TMT Site of Binding

Revised MWF & MLW February 16, 2024

For probe competition using TMT6.

DAY 1

A-1. START HERE if you are performing inhibitor competition

*don’t measure less than 2 mg of compound when making DMSO stocks.

1. Thaw 1 mg proteome aliquots on ice

*proteomes are in PBS with no phosphatase inhibitors

*proteome concentrations are 2.0 mg/mL to achieve 1 mg total using volumes specified below

2. Use 500 μL of proteome at 2mg/mL

3. Add 5 μL of 100x compound (in DMSO), vortex.

4. Incubate for specified time/temperature. Time may be adjusted based on compound.

5. Follow 3 and 4 in A-2

A-2. START HERE if you are only performing probe labeling

*don’t measure less than 2 mg of compound when making DMSO stocks.

1. Thaw 1 mg proteome aliquots on ice

*proteomes are in PBS with no phosphatase inhibitors

*proteome concentrations are 2.0 mg/mL to achieve 1 mg total using volumes specified below

2. Use 500 μL of proteome at 2mg/mL

3. Add 5 μL of 100x probe (in DMSO), vortex.

4 Incubate for specified time/temperature. Time may be adjusted based on probe.

B. Click chemistry

*When making a master click chemistry mix, add CuSO4  to master mix directly before addition to samples.

  - 1. Master Mix/ Sample

- Tag (200 μM):

  - 10 μL of 50x (10 mM, 2 mg/410 μL in DMSO) Biotin-N3 or desthiobiotin-N3

- Reducing Agent (1 mM):

  - 10 μL of 50x (50 mM, 7.17 mg/500 μL in H2O) TCEP (make fresh)

- Ligand (100 μM):

  - 33 μL of 16.7x (1.67 mM) TBTA. 1.67 mM = 20 μL 50x + 180 μL DMSO + 800 μL t-BuOH 8.55mg into 200 μL in DMSO gives 50x stock

- Catalyst (1 mM):

  - 10 μL of 50x (50 mM, 12 mg in 960 μL H2O) CuSO4 (Fisher Scientific CAS 7758-98-7)

  - 2. Add 63 μL of master-mix to each sample

  - 3. Vortex to mix and incubate at RT for 1 hour

C. MeOH/CHCl3 Extraction

1. To a 2-dram glass vial add:

a. 2 mL MeOH

b. 0.5 mL CHCl3

c. sample (500 μL)

2.  Wash sample tube with 1.5 mL H2O and add wash to dram vial, vortex. *Total volume 4.5 mL

3. Centrifuge 1,400 g x 3 min, 4°C; Allow dram vial to rest for 3 min or until upper layer is clear

4. Remove top and bottom layers, leaving protein interface

5. Resuspend protein disk in 600 μL MeOH and 150 μL CHCl3.

6. Transfer protein to screw-cap microcentrifuge tube (Sarstedt D-51588)

7. Wash dram vial with 600 μL H2O and add wash to screw-cap microcentrifuge tube, vortex.

8. Centrifuge 1,400 g x 3 min, rest until upper layer is clear

9. Remove top and bottom layers, leaving protein interface

10. Add 600 μL MeOH, sonicate 3 pulses (20% amplitude)

11. Centrifuge 14,000 g x 5 min to pellet protein, remove MeOH

D. Reduction, Alkylation

1. Re-suspend pellet in 500 μL of 6 M urea/25 mM ammonium bicarbonate (AmBic)

- AmBic (250 mM):

Weigh 0.198 g and resuspend in 10 mL Optima water

- Urea (6 M)

Weigh 3.6 g, add 1mL 250 mM AmBic, bring total volume to 10mL

2. Sonicate with probe sonicator 3 pulses (20% amplitude)

3. Add 5 μL 100x (1M, 15.4 mg/100 μL in H2O) DTT, giving 10 mM final (make fresh)

4. Incubate at 65C for 15 min, let cool to RT or cool briefly on ice

5. Add 40 μL 50x (0.5 M, 28 mg/300 μL in H2O) IA, giving 40 mM IA (make fresh)

6. Incubate RT for 30 min in the dark

E. MeOH/CHCl3 Extraction

1. To a 2-dram glass vial add:

a. 2 mL MeOH

b. 0.5 mL CHCl3

c. sample (500 μL)

2.  Wash sample tube with 1.5 mL H2O and add wash to dram vial, vortex. *Total volume 4.5 mL

3. Centrifuge 1,400 g x 3 min, 4°C; Allow dram vial to rest for 3 min or until upper layer is clear

4. Remove top and bottom layers, leaving protein interface

5. Resuspend protein disk in 600 μL MeOH and 150 μL CHCl3.

6. Transfer protein to screw-cap microcentrifuge tube (Sarstedt D-51588)

7. Wash dram vial with 600 μL H2O and add wash to screw-cap microcentrifuge tube, vortex.

8. Centrifuge 1,400 g x 3 min, rest until upper layer is clear

9. Remove top and bottom layers, leaving protein interface

10. Add 600 μL MeOH, sonicate 3 pulses (20% amplitude)

11. Centrifuge 14,000 g x 5 min to pellet protein, remove MeOH

F. Trypsin digestion

1. Re-suspend protein pellet in 500 μL 25 mM AmBic

2. Sonicate with probe sonicator 3 pulses (20% amplitude)

3. Reconstitute 20 μg Tryp-Lys-C (1 vial) with 100 μL of 25 mM AmBic.

4. Add 6.0 μg (30 μL) Tryp-Lys-C to sample and incubate at 37 C overnight.

DAY 2

- G. Desalt peptides (Reference: Pierce™ Peptide Desalting Spin Columns User Guide)

- a) Prepare materials

- 1. Conditioning/Washing Solution (0.1% TFA in water) — Each sample requires 1.5 mL for processing. Add 2 μL of trifluoroacetic acid to 2 mL water.

- 2. Elution Solution (0.1% TFA, 50% ACN) — Each sample requires 600 μL for elution. Add 2 μL of trifluoroacetic acid to 1 mL water and 1 mL acetonitrile. Mix well.

- b) Condition and Wash Column

- 1. Condition the spin columns 1. Remove the white tip from the bottom of the column and discard. Place the column into a 2 mL microcentrifuge tube.

- 2. Centrifuge at 5,000 × g for 1 minute to remove the solution (1:1 water/DMSO) and pack the resin material. Discard the liquid. Note: Do not exceed recommended centrifugation speeds.

- 3. Remove the top screw cap and load 300 μL of ACN into the column. Replace the cap, place the spin column back into a 2 mL microcentrifuge tube and centrifuge at 5,000 × g for 1 minute. Discard ACN and repeat the wash step.

- 4. Wash the spin column twice with 0.1% TFA in water, as described in step 3. The column is now conditioned and ready for use.

- c) Load and Wash samples

- 1. Dissolve 5-5,000 μg of digested sample in 300 μL of 0.1% TFA in water. Note: Peptide samples need to be completely dissolved and free of organic solvent (e.g., ACN, DMSO, etc.).

- 2. Place the spin column into a 2 mL microcentrifuge tube. Load 300 μL of the sample solution onto the column, replace the top cap and centrifuge at 3,000 × g for 1 minute. Discard the flowthrough.

- 3. Place the column into a 2 mL microcentrifuge tube. Load 220 μL of 0.1% TFA solution onto the column and centrifuge at 3,000 × g for 1 minute to collect the wash. Discard the eluate and repeat this step 2 more times for a total of 3 washes.

- d) Collect samples

- 1. Place the column into a lobind 2 mL microcentrifuge tube. Load 240 μL of the 50% ACN, 0.1% TFA solution and centrifuge at 3,000 × g for 1 minute to collect the eluate.

- 2. Apply another 240 μL of the 50% ACN, 0.1% TFA solution and centrifuge at 3,000 × g for 1 minute, collecting the eluate into the same microcentrifuge tube.

- 3. Evaporate the liquid contents of each sample tube to dryness using vacuum centrifugation (e.g., SpeedVac concentrator)

- 4. Re-suspend dry peptides in 30 μL 200 mM EPPS (pH 8.5)

- 5. Determine the peptide concentration and yield with a peptide quantitation assay.

- H. Pierce™ Quantitative Colorimetric Peptide Assay

- a) Prepare materials

1.Create standard curve of peptides using kit standard (1000 μg/mL-15.6 μg/mL) in 200 mM EPPS (pH 8.5)

- 2. Prepare samples by diluting peptides 20x into measurement range.

- 3. Pipette 5 μL of each standard or unknown sample in triplicate into 96-well microplate.

- 4. Add 45 μL of the WR to each well.

- 5. Cover plate with aluminum foil and incubate at 37°C for 15 minutes.

- 6. Read well absorbance at 480 nm with the script “peptide conc.” on the Clariostar.

- 7. Ensure standard curve linearity R2> 0.98

- 8. Calculate peptide concentration considering the 1:20 dilution.

WR

Reagent A: 22.5 μL/sample

Reagent B: 21.6 μL/sample

Reagent C: 0.9 μL/sample

*Prepare 20% more WR than needed

- I. TMT labeling

NOTE: RECORD TMT LOT NUMBER. Reporter ion isotopic distributions are unique to each lot number and are necessary for accurate data quantification.

Normalize peptides to 5 μg/μL in 200 mM EPPS (pH 8.5).

1. Aliquot 12 μL of 5 μg/μL peptides

2. To each aliquot, add 0.6 uL BSA standard peptide (BSA-SuTEx in -80C w/ TMT kits)

3. Transfer 10 μL of peptide to the TMT reagents

Final solution will be 15 μL (33% ACN) labeling at a 2:1 ratio (TMT: peptide, wt: wt).

4. Briefly vortex and centrifuge the PCR strip.

5. Incubate for 1 hour at RT with shaking at 500 rpm.

K. TMT Reaction Quench

- 1. Quench reactions by separately adding 1.0 μL of 5% hydroxylamine to each tube (final 0.3%)

5% hydroxylamine is 50 mg hydroxylamine/1.0mL 200mM EPPS (pH 8.5)

2. Incubate the quenched reaction for 15 min at RT with shaking at 1,200 rpm. *Do not quench for longer than 15 min. Hydroxylamine can hydrolyze probe-modified adducts on Y, S and T.

3. Add 1 µL of formic acid to each tube to acidify and neutralize the excess hydroxylamine. *Excess hydroxylamine can hydrolyze probe-modified adducts on Y, S and T.

4. Combine ALL TMT labeled samples into one low bind tube

5. Wash each labeling PCR strip tube 2 times with both 20 μL 50/50 ACN: H2O and 20 μL of EPPS (pH 8.5)

6. Snap freeze and SpeedVac for 30-90 min. to dry

J. Unenriched Proteomics

- 1. Resuspend dried peptides in 300 uL Optima water

- 2. Remove 5 uL of resuspension (5000 ng peptide) and add to 45 uL mobile A

- 2. Preform Stage tip cleanup

- 3. Dry the Stage tip eluate via SpeedVac

- 4. Reconstitute peptides in 50 uL mobile A for [FINAL] of 100ng/uL

- 5. Inject 3uL onto instrument (300ng) on unenriched TMT gradient (140 min)

- 6. Check Byonic result for labeling efficiency

DAY 3

- L. Enrichment for SuTEx modified peptides

- **For all decanting steps, only pour once. Pouring twice will result in bead loss.

- *** When pipetting avidin beads, use P1000 tips to allow slurry to transfer evenly, ensuring the amount of beads is equal in each tube.

- 1. Remove 100 μL aliquot of 50% avidin slurry per sample and place in 15 mL conical, (1 bed volume = 100 μL)

- 2. Wash beads by adding 10 mL of PBS, centrifuging at 1,400 x g for 1 min and decanting

- 3. Repeat for a total of 3 washes

- 4. Resuspend washed beads in 1 bed volume of PBS.

- 5. Add peptides to beads (295uL); washing the screw-cap tube with 1.0 mL PBS.

- 4. Add PBS for a total volume of 5.5 mL per sample.

- 5. Incubate RT, rotating for 1 hr

- 6. Spin 1,400 x g for 1 min to pellet; decant ONCE

- M. Wash beads/elution

- 1. Wash beads with 10 mL 25 mM AmBic

- 2. Spin 1,400 x g for 3 min to pellet; decant ONCE

- 3. Repeat for a total of 3 washes

- 4. Wash beads with 10 mL Optima water

- 5. Spin 1,400 x g for 3 min to pellet; decant ONCE

- 6. Repeat for a total of 3 washes

- 7. Transfer beads in 200 μL of Optima water to low-bind microcentrifuge tubes.

- 8. Spin 1,400 x g for 3 min to pellet

- 9. Remove supernatant (90%) with gel tip

- 10. Elute peptides:

- Add150 μL of elution buffer (50% ACN, 0.1% formic acid (FA)) to beads

- Incubate 3 minutes

- Centrifuge 1,400g x 3min

- Let rest 1 min

- Transfer 140 μL (of the original 150 μL) of eluate to a new low bind using gel loading pipette tip

- Repeat for a total of 3 elutions, adding 140uL to begin elutions 2&3

11. Transfer all eluent to a bio-rad biospin column (to trap any beads that accidentally transferred), collect in clean microcentrifuge tube by centrifugation (1,400 x g for 3 min)

12.Snap freeze and speedvac

13. Re-suspend peptides in 50 μL of mobile phase A (LC-MS water + 0.1% FA)

Stage Tip cleanup of peptides

A. Protocol to prepare the StageTip

Materials and Assembly:

Empore Extraction Disks:

Octadecyl (C18)-bonded silica, part no: 2215, Vendor: 3M, St. Paul, MN

200 μl Pipette tips

Needle: Kel-F Hub (KF), point style 3, gauge 16(1.19mm) part no: 90516

Plunger Assembly: N, RN, LT, LTN for model 1702 (25 μl)

1. Place one Empore extraction disk into a petri dish.

2. Use the Needle to pick up the stationary phase from the disk.

3. Use the plunger to insert into the 200 μl pipette tip.

B. C18-StageTips for clean-up and concentration

  - 1. Condition column by wetting disk with 20 μL methanol (0.9G for 1 min) a. Do not let tips dry completely

  - 2. Add 20 μL Buffer B (80% ACN + 0.1% FA) to stage tip. Spin at 900 x g for 1 min.

  - 3. Add 20 μL Buffer A (Water + 0.1% FA) to stage tip. Spin at 900 x g for 1 min.

  - 4. Load sample directly to the disc using gel loading tip. Spin at 900 x g for 1 min.

  - 5. Wash using 20 μL Buffer A. Spin at 900 x g for 1 min.

*Repeat step three times. This step might take longer than 1 minute, but check it periodically and ensure the stage tip does not dry.

6. Switch to low-bind tube.

7. Elute using 20 μL buffer B. Spin at 900 x g for 1 min.

*Repeat step three times. This step might take longer than 1 minute, but check it periodically and ensure the stage tip does not dry.

8. Snap freeze peptides and speedvac (approx. 30min)

9. Re-suspend peptides in 25 μL of mobile phase A (LC-MS water + 0.1% FA) and store in 2 amber vials (12.5 uL ea.)

10. Analyze 3 μL of sample on LC-MS using 20231014_EclipseSuTExTMT85_ms2_FINAL.meth method or equivalent on Exploris.